}ULASER}.ul on

.cw10

.rm6.5"

.op

TO:       Bill Henry

Rex Debolt

FROM:     A.B. White

SUBJECT:  ST-8000 BAUDOT TRANSLATION TABLE

DATE:     April 24, 1990

.fo ABW  4-24-90  stmv14c.doc

1.  INTRODUCTION

A special version of our standard ST-8000 firmware is now available which

permits a user to scramble or encode 5 bit characters before transmission and

unscramble or decode these 5 bit codes during reception.  A 32 byte conversion

table in EPROM U10 defines the conversion for both transmit and receive.  By

making changes in this table, a user may convert from standard 5 level BAUDOT

code, for example, to some new intermediate code where some number of the

characters are translated into another 5 bit character.

The code conversion is active whenever the REGEN control is set to A/B.

2.  CONVERSION TABLE

A single table is used for both transmit encoding and receive decoding.

During transmission, the 5 level character received from the attached terminal

device on the TXD input is used to calculate an offset into a 32 byte table.

The 5 bit code stored at this offset location is transmitted by the ST-8000 to

the modem transmit output.

During receive, the same conversion table is used in the same manner described

above.  The 5 bit character received by the modem is used to calculate an

offset into the 32 byte conversion table.  The 5 bit code stored at this

offset location is sent to the RXD output of the ST-8000.  Note that it is

possible to have more than one encoded byte convert into the same decoded

character.

The default table, shown below, will transmit each 5 bit character with the

same 5 bit code because the bytes in the conversion table are stored

sequentially from 00H to 1FH.

0100                   ORG     100H              ; CONVERSION TABLE IN U10

0100 00010203  BBTAB:  DB      00H,01H,02H,03H   ; 00 01 02 03

0104 04050607          DB      04H,05H,06H,07H   ; 04 05 06 07

0108 08090A0B          DB      08H,09H,0AH,0BH   ; 08 09 0A 0B

010C 0C0D0E0F          DB      0CH,0DH,0EH,0FH   ; 0C 0D 0E 0F

0110 10111213          DB      10H,11H,12H,13H   ; 10 11 12 13

0114 14151617          DB      14H,15H,16H,17H   ; 14 15 16 17

0118 18191A1B          DB      18H,19H,1AH,1BH   ; 18 19 1A 1B

011C 1C1D1E1F          DB      1CH,1DH,1EH,1FH   ; 1C 1D 1E 1F

NOTE: table changes do not effect the power on checksum calculation.

ST-8000  V1.4C Checksum:  U10 0100

U11 BC00